KUREAS-SEQ

Process Operational Logic

Sequence Modeling for Hazards

User Help Guide   |   v1.1

KUREAS — Knowledgebase with Uncertainty for Risk Engineering Analysis of Systems

# Contents

1.  Overview	3

2.  Recommended Workflow	3

3.  Step 1 — Load the KUREAS-HA File	4

4.  Step 2 — Create System Modules	5

5.  Step 3 — Develop Facility Scenarios (HTFW Script)	6

6.  Step 4 — Evaluate Outcomes via How the Facility Fails	7

7.  Saving and Loading Projects	8

8.  Settings	9

9.  Reporting	10

10.  Quick-Reference Card	11

# 1.  Overview

KUREAS-SEQ is a standalone, browser-based tool for developing Process Operational Logic (POL) diagrams and scripted facility scenarios in support of probabilistic risk assessment (PRA). It bridges the qualitative hazard analysis performed in KUREAS-HA with the quantitative fault-tree models maintained in KUREAS-SYS, allowing analysts to describe how a facility responds to initiating events and to compute sequence frequencies.

The module operates in scripting modes:

- HTFW Script mode — Write a plain-language success-logic script (FACILITY WORKS …) for each initiating event. The diagram is generated automatically from the script.

How the Facility Fails function, which converts a scenario into an event-tree structure, expands each system/function node to its cut sets, and computes the conditional probability and frequency of every sequence end state.

Standalone file — no installation required. Open KUREAS-SEQ.html in any modern browser. All data is processed locally; nothing is sent to a server.

# 2.  Recommended Workflow

Follow these four steps in order to build a complete sequence analysis.

| Step | Action | Result |
| --- | --- | --- |
| 1 | Load a KUREAS-HA .KNOW_HA file via Load HA | Initiating events (IEs) and operating states populate the left panel |
| 2 | Create system modules by loading KUREAS-SYS .KNOW_FTL files | Each module stores cut sets and basic event probabilities for one system/function |
| 3 | Write a FACILITY WORKS script for each initiating event | Describes facility success logic; the diagram renders in real time |
| 4 | Click How the Facility Fails to evaluate outcomes | Event tree, sequence probabilities, and cut sets are displayed |

The first three steps build the project; step 4 is the analysis output. Steps 2–4 are iterative — scripts and system associations can be updated at any time.

# 3.  Step 1 — Load the KUREAS-HA File

The hazard analysis file defines all initiating events, their associated operating states (facility operating states, or FOS), and the initiating event frequencies. KUREAS-SEQ requires this context before scenario scripts can be written.

## 3.1  Procedure

- Click Load HA in the header toolbar.

- Select a KUREAS-HA project file (.KNOW_HA). The file browser accepts .KNOW_HA, .KNOW, and .know extensions.

- A FOS Selector dialog appears. Choose the Facility Operating State to use as the reference for this analysis session. The selection determines which hazard–state combinations are treated as initiating events.

- Click Confirm. The left panel populates with all hazards that are marked as Included for the chosen FOS.

## 3.2  What Loads

- Hazard list — All hazards from the HA, organized in their category hierarchy.

- Operating states — The full set of FOS defined in the HA project.

- Initiating event frequencies — Occurrence model data (point estimate, lognormal, gamma, CNI, or NUREG-2169 fire) for each included hazard–state combination. Frequencies are displayed next to the IE in the left panel and embedded in the diagram.

- Facility name — Pre-fills the project name field if the project has not yet been named.

IE scripts are automatically initialized for every included hazard when the HA file is loaded. Each script begins with a FACILITY WORKS header line and an empty comment.

## 3.3  Re-loading an Updated HA File

If the hazard analysis is revised, click Load HA again and select the updated file. A selective merge dialog appears showing new and changed items. Choose which items to import; existing scripts are preserved unless you explicitly replace them.

# 4.  Step 2 — Create System Modules

System modules link a function name used in the HTFW script to the quantitative fault-tree data produced by KUREAS-SYS. Each module stores the system’s cut sets and basic event probabilities so that How the Facility Fails can compute conditional failure probabilities.

## 4.1  Opening the Module Panel

Click System/Function and End State Elements (the purple panel below the left hazard panel) to expand it. If the panel is hidden, click the toggle button labeled System/Function and End State Elements in the script editor area.

## 4.2  Creating a Module

- Click + New in the module panel header.

- Name — Enter a short identifier using only letters, digits, underscores, or hyphens (e.g., ECCS or RHR_Pump). This identifier is the name you will reference in HTFW scripts.

- Type — Choose Sys / Function or End State.

- Sys / Function — Represents a system or safety function that either succeeds (YES) or fails (NO) in response to an initiating event.

- End State — Represents a final outcome of the scenario. Assign an Outcome (Positive, Negative, or Unanalyzed) so the diagram can colour-code the result.

- Display Name — Optional human-readable label shown in the diagram and event tree.

- KUREAS-SYS FTL File — Click Load System and select the .KNOW_FTL file exported from KUREAS-SYS for this system. The file embeds cut sets and basic event data directly into the project.

- Click Save.

## 4.3  Loading a System (.KNOW_FTL File)

When you click Load System (either in the module panel or on a Function node in the POL editor), a file picker opens. Select the corresponding SYS_SystemName.KNOW_FTL file. After loading, a green confirmation line reports the number of cut sets and basic events ingested and the load time.

Cut sets and basic event probabilities from the .KNOW_FTL file are saved inside the .KNOW_SEQ project file. Collaborators do not need access to the original .KNOW_FTL files to open the project and run How the Facility Fails.

## 4.4  Module Chips

Saved modules appear as colored chips at the top of the module panel. Purple chips are Sys/Function modules; End State chips use the color of their outcome (green = positive, red = negative, grey = unanalyzed). Click a chip to edit the module; click the x to delete it.

# 5.  Step 3 — Develop Facility Scenarios (HTFW Script)

The HTFW (How the Facility Works) script is a plain-language description of the conditions under which the facility succeeds following an initiating event. KUREAS-SEQ parses this script to generate a flowchart and, ultimately, the sequence event tree.

## 5.1  Selecting an Initiating Event

Click any hazard row in the left panel to select it as the active IE. The center pane splits into a script editor (left) and a live diagram preview (right). Each IE has its own independent script.

## 5.2  Script Structure

Every script begins with a header line that names the initiating event:

FACILITY WORKS(IE_Name) =

Below the header, write one or more conditional statements. Lines beginning with # are comments and are ignored by the parser. Each statement must end with a semicolon.

## 5.3  Grammar Reference

| Statement form | Meaning |
| --- | --- |
| IF(F1 THEN ENDSTATE(Safe); | If function F1 succeeds (YES branch), the scenario reaches end state Safe |
| IF(F1 AND F2) THEN ENDSTATE(Safe); | Both F1 and F2 must succeed for end state Safe |
| IF(NOT F1 THEN ENDSTATE(CoreDamage); | If F1 fails (NO branch), the scenario reaches end state CoreDamage |
| IF(F1 AND NOT F2) THEN ENDSTATE( Degraded); | F1 succeeds and F2 fails |
| IF(F1) THEN FAILED(BE1, BE2) AND ENDSTATE(Partial_CD); | F1 succeeds but basic events BE1 and BE2 are forced to probability 1.0 |
| system = CNI(mean_value); | Set a top event split fraction to a constrained non-informative distribution with a mean specified by “mean_value.” |
| system = BETA(alpha, beta); | Set a top event split fraction to a Beta distribution. |
| FAILED(BE1); | Global override: BE1 is forced to P=1.0 for all sequences in this IE script |

## 5.4  Identifiers

- Function / system names — Must match the Name field of a module (e.g., ECCS, RHR_Pump). The parser is case-sensitive.

- End state names — Should match the Name field of an End State module for colour coding. If no matching module exists, the end state is drawn as unanalyzed (grey).

- Basic event names in FAILED( ) — Must exactly match basic event names in the system’s loaded cut sets.

## 5.5  Example Script

# Scenario: Loss of Coolant Accident (small-break) FACILITY WORKS(SBLOCA) = # Success path: ECCS actuates and maintains core cooling IF(ECCS AND RHR) THEN ENDSTATE(OK); # ECCS fails but operator action compensates IF(ECCS AND NOT RHR) THEN ENDSTATE(OK_Degraded); # ECCS fails entirely IF(NOT ECCS) THEN ENDSTATE(Core_Damage);

## 5.6  Live Diagram

As you type, click Update Diagram to re-render the flowchart. The diagram uses the following visual conventions:

| Node type | Shape / Colour |
| --- | --- |
| Initiating Event | Hexagon — red/amber background |
| System / Function | Rectangle — blue background; shows display name and description |
| End State (Positive) | Rounded rectangle — green background |
| End State (Negative) | Rounded rectangle — red background |
| End State (Unanalyzed) | Rounded rectangle — grey background |
| FAILED events | Hexagon — orange background; lists forced-failure events |

## 5.7  Properties Panel in the Diagram

The diagram renders a collapsible Properties sidebar showing each end state and function node. Use this panel to:

- Assign or change the outcome (positive / negative / unanalyzed) of an end state.

- Load System — attach a .KNOW_FTL file to a specific function node directly from the diagram without returning to the module panel.

- Add a description to any node for richer documentation.

## 5.8  Diagram Actions

| Action | How to perform it |
| --- | --- |
| Zoom in / out | Scroll wheel over the diagram |
| Copy diagram to clipboard | Click the Copy icon in the diagram toolbar |
| Download diagram as PNG | Click the Download icon in the diagram toolbar |
| Full-screen view | Click the Full-screen icon in the diagram toolbar |

# 6.  Step 4 — Evaluate Outcomes via How the Facility Fails

How the Facility Fails (HTF) converts the active HTFW script (or the active POL diagram) into an event tree, expands each system/function node to its fault-tree cut sets, computes the conditional probability and frequency of each sequence, and identifies dominant failure contributors.

## 6.1  Launching the Function

Ensure an initiating event (or a POL diagram) is selected in the left panel. Click the How the Facility Fails button in the header toolbar (dark red). The analysis modal opens.

## 6.2  Modal Layout

| Panel | Content |
| --- | --- |
| Left — Sequence Tree | Hierarchical tree showing the initiating event, each system/function branch (YES = success, NO = failure), and the resulting end state for each sequence |
| Right — Event Tree Canvas | Classic event-tree diagram with the IE on the left, top events (one per system/function) across the top, and end states on the right. IE frequency and sequence frequencies are annotated. Copy or Download buttons export the diagram. |

## 6.3  Sequence Tree Navigation

- Expand / Collapse — Click the triangle next to any branch node to expand or collapse its sub-sequences.

- Expand All / Collapse All — Buttons at the top of the tree panel.

- Each sequence row shows: end state name, conditional probability of that sequence (P), and sequence frequency (f = P × IE frequency).

- Sequences without complete probability data (system not loaded) display “—” for probability.

## 6.4  System Expansion (Cut Sets)

Click any NO branch (failed system) in the sequence tree to expand it. The expansion shows:

- System name and source file name.

- Cut sets — Numbered list sorted by cut set probability (highest first). Each cut set shows: cut set index, list of contributing basic events, and cut set probability.

- FAILED() overrides — Basic events named in FAILED() statements are shown at probability 1.0 regardless of their stored values. The event tree clearly marks affected sequences.

## 6.5  Quantification Method

The conditional probability for each NO branch is the system failure probability, calculated using either MCUB (Minimal Cut-set Upper Bound) or BDD (Binary Decision Diagram), depending on the project setting. The method is configurable in Settings. MCUB is faster and suitable for most analyses; BDD provides exact inclusion-exclusion results.

## 6.6  IE Frequency

The IE frequency shown in the modal is derived from the KUREAS-HA data for the selected hazard and FOS. It is displayed on the initiating event node in the diagram and used to compute absolute sequence frequencies (f = P_{sequence} × f_{IE}).

## 6.7  Reading the Event Tree Canvas

- The leftmost column shows the initiating event name and frequency.

- Each column across represents one system/function (top event). The column header shows the function name.

- Vertical lines represent success (upward branch) or failure (downward branch) at each top event.

- The rightmost column lists end states with sequence probability and frequency.

- Copy — Copies the event tree as a PNG image to the clipboard.

- Download — Saves the event tree as a PNG file.

## 6.8  Warnings

| Warning | Meaning |
| --- | --- |
| Yellow highlight on YES branch | The associated system has a failure probability of 1.0; the success probability is effectively zero |
| Orange highlight on NO branch | The system is fully failed (failure probability = 1.0) |
| — (dash) for probability | The system module has not been loaded; failure probability cannot be computed |

# 7.  Saving and Loading Projects

| Action | Detail |
| --- | --- |
| Save SEQ | Click Save SEQ in the header. The project is downloaded as ProjectName.KNOW_SEQ (JSON). All IE scripts, module definitions, loaded cut sets, POL diagrams, and project settings are saved in this single file. |
| Load SEQ | Click Load SEQ. If the workspace already contains data, a selective merge dialog shows new and changed items so you can choose what to import without overwriting existing work. |
| Load HA | Can be reloaded at any time to refresh hazard data. Existing scripts are preserved. |
| File extension | .KNOW_SEQ |
| Unsaved-changes indicator | The browser will prompt before navigation if there are unsaved changes. |

Windows users: Save files frequently. The browser does not auto-save. Use a consistent folder so that .KNOW_SEQ and .KNOW_FTL files remain easy to locate when reloading systems.

# 8.  Settings

Click the Settings button (⚙) in the header to open the Settings modal. Settings are divided into two groups:

| Setting group | Detail |
| --- | --- |
| GUI settings (browser-level) | Font family, font size, background theme (light / dark / gray), arc label size, number of significant digits displayed. Stored in browser localStorage; persist across sessions on the same browser. |
| Project settings (project-level) | Node colours for each node type, quantification method (MCUB or BDD), display digits, document settings (page size, font, font size for reports). Saved inside the .KNOW_SEQ file; travel with the project. |

# 9.  Reporting

Click Generate Report in the header. The report is produced as a Word-compatible .doc file and downloaded automatically.

The report includes:

- Cover information: facility name, generation date.

- Summary table: POL name, hazard, operating state, number of nodes.

- Per-POL sections: POL diagram image (PNG), node details table, and rich-text detailed description (if any).

Report formatting options (page size, font, font size) are configured in the Project Settings tab of the Settings modal.

# 10.  Quick-Reference Card

## Header Toolbar Buttons

| Button | Function |
| --- | --- |
| Load SEQ | Open a .KNOW_SEQ project file |
| Save SEQ | Download the current project as .KNOW_SEQ |
| How the Facility Fails | Open the HTF analysis modal for the selected IE or POL diagram |
| Load HA | Load a KUREAS-HA .KNOW_HA file to import initiating events and FOS |
| System Info (?) | Display version and project summary information |
| Filter (▽) | Filter the left-panel hazard list by operating state or hazard category |
| Generate Report | Download a Word-compatible report (.doc) for the project |
| Settings (⚙) | Open GUI and project settings |

## Script Grammar Summary

# Header (required, one per script) FACILITY WORKS(IE_Name) = # Standard conditional (succeeds) IF(Func1) THEN ENDSTATE(EndStateName); # Conjunction IF(Func1 AND Func2) THEN ENDSTATE(EndStateName); # Failure branch IF(NOT Func1) THEN ENDSTATE(EndStateName); # Failure branch with forced basic event IF(Func1) THEN FAILED(BasicEvent1, BasicEvent2) AND ENDSTATE(EndStateName); # Global basic event override FAILED(BasicEventName); # Assign a split fraction probability Sys1 = CNI(1E-4); # Comments (entire line, anywhere in the body) # This line is ignored

## Node Color Legend

| Node type | Default colour |
| --- | --- |
| Initiating Event | Light red (#fecaca) |
| Function / System | Light blue (#e0f2fe) |
| Transition | Light purple (#e9d5ff) |
| End State | Light green (#bbf7d0) |

## File Types

| Extension | Description |
| --- | --- |
| .KNOW_HA | KUREAS-HA project file — source of initiating events and FOS |
| .KNOW_FTL | KUREAS-SYS system file — source of cut sets and basic event data |
| .KNOW_SEQ | KUREAS-SEQ project file — scripts, modules, POL diagrams, system data |